Computer Networks

Practical Lab Manual — Step-by-Step Guide

B.Sc. (H) Computer Science | Cisco Packet Tracer

This manual covers all 9 practicals as per the NEP UGCF 2022 guidelines (DSC12/DSC06/GE 6a), effective from Academic Year 2024-25. Each practical includes objective, required devices/settings, and detailed numbered steps to follow inside Cisco Packet Tracer.

Practical 1 — Basic Network Commands & Network Configuration Commands

Objective

To study and execute basic network diagnostic commands and understand network configuration commands using Cisco Packet Tracer's Command Prompt and CLI.

Tools Required

Cisco Packet Tracer (any version ≥ 7.0)

1 PC, 1 Router, 1 Switch (for CLI practice)

Part A — Basic Network Commands on a PC

Step-by-step instructions:

Open Cisco Packet Tracer. Click File → New to create a new project.

From the bottom-left device panel, drag one PC onto the workspace.

Click on the PC to open its properties window.

Go to the Desktop tab and click Command Prompt.

Type the following command to view the IP configuration of the PC:

ipconfig

Continue with these commands:

ipconfig /all          (Shows MAC address, DNS, Gateway, etc.)

ping 127.0.0.1         (Loopback test — checks if TCP/IP stack works)

ping <destination IP>  (Tests reachability of another device)

tracert <IP>           (Shows hop-by-hop path to destination)

arp -a                 (Shows ARP cache — IP to MAC mappings)

netstat                (Shows active connections and ports)

Part B — Router Configuration Commands (CLI)

Drag a Router (e.g., 2911) onto the workspace.

Click on the Router → go to the CLI tab.

Press Enter to get to the Router> prompt.

Enter Privileged EXEC mode:

Router> enable

5. Enter Global Configuration mode:

Router# configure terminal

6. Set the router hostname:

Router(config)# hostname MyRouter

7. Set a privileged EXEC password:

MyRouter(config)# enable secret cisco123

8. View running configuration:

MyRouter# show running-config

9. View interfaces status:

MyRouter# show ip interface brief

10. Save configuration:

MyRouter# write memory

📝 Note: In Packet Tracer, 'ping' and 'tracert' only work after assigning IP addresses to devices and connecting them with cables.

Practical 2 — PC to PC Communication

Objective

To establish direct communication between two PCs using a crossover cable and verify connectivity using ping.

Devices Required

2 PCs

1 Copper Cross-Over cable

Steps

Open Cisco Packet Tracer and create a new blank project.

From the End Devices panel (bottom-left), drag PC0 and PC1 onto the workspace.

Click the Connections icon (lightning bolt) at the bottom. Select Copper Cross-Over cable (shown as a dashed line).

Click on PC0, select FastEthernet0 port. Then click on PC1 and select its FastEthernet0 port. The cable should appear between them.

Wait for the link lights on both ends to turn GREEN (this may take a few seconds in Packet Tracer).

Click on PC0 → Desktop tab → IP Configuration.

Set the following for PC0:

IP Address   : 192.168.1.1

Subnet Mask  : 255.255.255.0

Gateway      : (leave blank for now)

8. Click on PC1 → Desktop tab → IP Configuration and set:

IP Address   : 192.168.1.2

Subnet Mask  : 255.255.255.0

9. Click on PC0 → Desktop → Command Prompt and type:

ping 192.168.1.2

10. You should see 4 successful replies:

Reply from 192.168.1.2: bytes=32 time<1ms TTL=128

11. Repeat from PC1 — ping PC0's address to verify bidirectional communication:

ping 192.168.1.1

📝 Note: If the ping fails, check that both link lights are green, IP addresses are in the same subnet, and a cross-over (not straight-through) cable was used for direct PC-to-PC connection.

Practical 3 — Star Topology Using Hub and Switch

Objective

To create a Star topology network using both a Hub and a Switch and observe the difference in data transmission behavior.

Devices Required

4 PCs

1 Hub (for Part A)

1 Switch — 2960 (for Part B)

Copper Straight-Through cables

Part A — Star Topology with Hub

Open a new Packet Tracer project.

From the Network Devices → Hubs panel, drag a Generic Hub onto the workspace.

From End Devices, drag 4 PCs (PC0 to PC3) around the Hub.

Select Copper Straight-Through cable. Connect each PC's FastEthernet0 port to any port on the Hub.

Assign IP addresses to each PC (Desktop → IP Configuration):

PC0: 192.168.1.10 / 255.255.255.0

PC1: 192.168.1.11 / 255.255.255.0

PC2: 192.168.1.12 / 255.255.255.0

PC3: 192.168.1.13 / 255.255.255.0

6. Switch to Simulation Mode (bottom-right clock icon).

7. From PC0, send a ping to PC1. Observe that the packet is broadcast to ALL ports (Hub flooding behavior).

Part B — Star Topology with Switch

Create a new project. From Network Devices → Switches, drag a Cisco 2960 Switch onto the workspace.

Add 4 PCs and connect each to the switch using Straight-Through cables.

Assign the same IP addresses as in Part A.

Switch to Simulation Mode. From PC0, ping PC1.

Observe that the packet goes ONLY to PC1 (after MAC address learning) — not broadcast to all PCs.

Check the switch MAC table using CLI: click the Switch → CLI tab:

Switch> enable

Switch# show mac address-table

7. Observe that the switch has learned which MAC address is on which port.

📝 Note: Hub = broadcasts to all ports (Layer 1 device). Switch = sends only to the correct port using MAC address table (Layer 2 device). This is the key difference.

Practical 4 — Bus, Ring, Tree, Hybrid, and Mesh Topologies

Objective

To create and understand different network topologies: Bus, Ring, Tree, Hybrid, and Mesh.

Part A — Bus Topology

Open a new project. Add 4 PCs in a horizontal line.

Connect PC0–PC1 with a Copper Cross-Over cable.

Connect PC1–PC2 with another Cross-Over cable.

Connect PC2–PC3 with another Cross-Over cable.

Assign IPs: 192.168.1.1 through 192.168.1.4 (all /24).

Use Simulation Mode to observe how data flows along the single shared line.

📝 Note: True bus topology uses a shared coaxial cable; in Packet Tracer, we simulate it with chained connections.

Part B — Ring Topology

Add 4 PCs arranged in a circle.

Connect: PC0→PC1→PC2→PC3→PC0 using Copper Cross-Over cables, forming a closed ring.

Assign IPs: 192.168.2.1 through 192.168.2.4.

In Simulation Mode, observe data traveling around the ring until it reaches the destination.

Part C — Tree Topology

Add 1 Root Switch (2960), 2 Intermediate Switches, and 4 PCs (2 per intermediate switch).

Connect each intermediate switch to the root switch using Straight-Through cables.

Connect 2 PCs to each intermediate switch.

Assign IPs: 192.168.3.1 through 192.168.3.4.

This creates a hierarchical tree structure.

Part D — Mesh Topology

Add 4 PCs onto the workspace.

Connect EVERY PC to EVERY other PC using Cross-Over cables.

With 4 PCs, you need: PC0↔PC1, PC0↔PC2, PC0↔PC3, PC1↔PC2, PC1↔PC3, PC2↔PC3 (6 cables total).

Assign IPs: 192.168.4.1 through 192.168.4.4.

Observe that each PC has multiple paths to reach others — this is full mesh redundancy.

Part E — Hybrid Topology

Combine two topologies: create a Star (switch + 3 PCs) and a Bus (3 PCs chained).

Connect the last PC of the bus to the switch using a Straight-Through cable.

Assign IPs in the same subnet: 192.168.5.1 through 192.168.5.6.

Ping across both segments to verify connectivity.

📝 Note: Label each topology using the 'Place Note' tool (the A icon in the toolbar) for easy identification in your saved file.

Practical 5 — Initial Switch Configuration

Objective

To perform initial configuration of a Cisco 2960 Switch including hostname, passwords, banner, and management IP.

Devices Required

1 Cisco 2960 Switch

1 PC (for verification)

Console cable + Straight-Through cable

Steps

Open Packet Tracer and add a 2960 Switch and one PC.

Connect the PC to the Switch using a Straight-Through cable.

Click on the Switch and go to the CLI tab. Press Enter.

Enter Privileged EXEC mode:

Switch> enable

5. Enter Global Configuration mode:

Switch# configure terminal

6. Set the hostname:

Switch(config)# hostname SW1

7. Set an enable (privileged) password:

SW1(config)# enable secret Admin@123

8. Set a console password:

SW1(config)# line console 0

SW1(config-line)# password console123

SW1(config-line)# login

SW1(config-line)# exit

9. Set a VTY (Telnet) password for remote access:

SW1(config)# line vty 0 15

SW1(config-line)# password telnet123

SW1(config-line)# login

SW1(config-line)# exit

10. Set a management IP address on VLAN 1:

SW1(config)# interface vlan 1

SW1(config-if)# ip address 192.168.1.100 255.255.255.0

SW1(config-if)# no shutdown

SW1(config-if)# exit

11. Add a login banner (Message of the Day):

SW1(config)# banner motd # Authorized Access Only #

12. Save the configuration:

SW1# write memory

13. Verify with:

SW1# show running-config

SW1# show version

SW1# show interfaces

📝 Note: The management IP on VLAN 1 allows remote Telnet/SSH management of the switch from any PC on the network.

Practical 6 — Initial Router Configuration

Objective

To perform initial configuration of a Cisco 2911 Router including hostname, interface IPs, passwords, and routing.

Devices Required

1 Cisco 2911 Router

2 PCs (one on each side)

Straight-Through cables

Steps

Open Packet Tracer. Add a Cisco 2911 Router and 2 PCs.

Connect PC0 to Router's GigabitEthernet0/0 using a Straight-Through cable.

Connect PC1 to Router's GigabitEthernet0/1 using a Straight-Through cable.

Click on the Router → CLI tab. Press Enter.

Enter Privileged EXEC mode:

Router> enable

6. Enter Global Configuration mode:

Router# configure terminal

7. Set the hostname:

Router(config)# hostname R1

8. Configure GigabitEthernet 0/0 (for PC0's network):

R1(config)# interface GigabitEthernet0/0

R1(config-if)# ip address 192.168.1.1 255.255.255.0

R1(config-if)# no shutdown

R1(config-if)# exit

9. Configure GigabitEthernet 0/1 (for PC1's network):

R1(config)# interface GigabitEthernet0/1

R1(config-if)# ip address 192.168.2.1 255.255.255.0

R1(config-if)# no shutdown

R1(config-if)# exit

10. Set the enable secret:

R1(config)# enable secret Router@123

11. Set VTY password:

R1(config)# line vty 0 4

R1(config-line)# password vty123

R1(config-line)# login

R1(config-line)# exit

12. Add banner:

R1(config)# banner motd # Router R1 - Authorized Only #

13. Assign IPs to the PCs:

PC0: IP=192.168.1.10, Mask=255.255.255.0, Gateway=192.168.1.1

PC1: IP=192.168.2.10, Mask=255.255.255.0, Gateway=192.168.2.1

14. Save the router config:

R1# write memory

15. Test: from PC0, ping PC1 (192.168.2.10). The router forwards the packet between networks.

16. Verify router status:

R1# show ip interface brief

R1# show running-config

📝 Note: The 'no shutdown' command is essential — Cisco router interfaces are administratively DOWN by default.

Practical 7 — Client-Server Network

Objective

To implement a Client-Server network using a DNS server and HTTP server in Cisco Packet Tracer.

Devices Required

1 Server (from End Devices panel)

2 PCs (clients)

1 Switch (2960)

Straight-Through cables

Steps — HTTP Server Setup

Open a new Packet Tracer project.

Drag 1 Server and 2 PCs onto the workspace.

Add a 2960 Switch and connect all three devices to it using Straight-Through cables.

Click on the Server → Desktop → IP Configuration and assign:

IP Address   : 192.168.1.100

Subnet Mask  : 255.255.255.0

Gateway      : 192.168.1.1

5. Click on the Server → Services tab.

6. Click HTTP in the left panel. Ensure HTTP is turned ON.

7. You can edit the index.html content that the server will serve.

8. Click DNS in the left panel. Turn ON the DNS service.

9. Add a DNS record:

Name    : www.myserver.com

Address : 192.168.1.100

10. Assign IPs to Client PCs:

PC0: IP=192.168.1.10, Mask=255.255.255.0, Gateway=192.168.1.1, DNS=192.168.1.100

PC1: IP=192.168.1.11, Mask=255.255.255.0, Gateway=192.168.1.1, DNS=192.168.1.100

11. On PC0 → Desktop → Web Browser.

12. In the URL bar, type:

http://192.168.1.100

13. The server's web page should load. Also try:

http://www.myserver.com

14. On PC0 → Desktop → Command Prompt, test DNS resolution:

ping www.myserver.com

15. Observe that the domain name resolves to 192.168.1.100.

📝 Note: You can also enable FTP service on the server and test file upload/download from a PC using the PC's Desktop → FTP client.

Practical 8 — Connection Between Devices Using a Router (Inter-Network Communication)

Objective

To connect two different networks using a router and verify inter-network communication.

Devices Required

1 Cisco 2911 Router

2 Cisco 2960 Switches

4 PCs (2 per network)

Straight-Through cables

Network Plan

Network A : 192.168.10.0/24  (PC0: .10, PC1: .11,  Gateway: .1)

Network B : 192.168.20.0/24  (PC2: .10, PC3: .11,  Gateway: .1)

Steps

Open a new Packet Tracer project.

Add Router R1 (2911), Switch SW1, Switch SW2, PC0, PC1, PC2, PC3.

Connect PC0 and PC1 to SW1 using Straight-Through cables.

Connect PC2 and PC3 to SW2 using Straight-Through cables.

Connect SW1 to Router's GigabitEthernet0/0.

Connect SW2 to Router's GigabitEthernet0/1.

Configure Router R1 — click Router → CLI:

Router> enable

Router# configure terminal

Router(config)# interface GigabitEthernet0/0

Router(config-if)# ip address 192.168.10.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface GigabitEthernet0/1

Router(config-if)# ip address 192.168.20.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# end

Router# write memory

8. Configure IP addresses on all PCs (Desktop → IP Configuration):

PC0: 192.168.10.10  /24  GW: 192.168.10.1

PC1: 192.168.10.11  /24  GW: 192.168.10.1

PC2: 192.168.20.10  /24  GW: 192.168.20.1

PC3: 192.168.20.11  /24  GW: 192.168.20.1

9. Test same-network ping (should work without router):

PC0> ping 192.168.10.11   (ping PC1)

10. Test cross-network ping (router needed):

PC0> ping 192.168.20.10   (ping PC2)

11. Verify with:

Router# show ip route

Router# show ip interface brief

📝 Note: If cross-network ping fails, verify the default gateway is correctly set on all PCs. Each PC must point to the router's interface on its own subnet.

Practical 9 — Remote Desktop Sharing Within LAN Connection

Objective

To perform remote desktop access within a LAN using Cisco Packet Tracer's built-in remote access simulation feature (Telnet to Switch/Router or PC remote access).

Devices Required

2 PCs

1 Switch (2960)

Straight-Through cables

Part A — Telnet Remote Access to a Switch

Open Packet Tracer. Add a 2960 Switch and 2 PCs. Connect both PCs to the switch.

Assign IPs:

PC0 : 192.168.1.10 / 255.255.255.0

PC1 : 192.168.1.11 / 255.255.255.0

3. Configure the Switch with a management IP and VTY password (from Practical 5):

Switch> enable

Switch# configure terminal

Switch(config)# interface vlan 1

Switch(config-if)# ip address 192.168.1.100 255.255.255.0

Switch(config-if)# no shutdown

Switch(config-if)# exit

Switch(config)# line vty 0 15

Switch(config-line)# password remote123

Switch(config-line)# login

Switch(config-line)# exit

Switch(config)# enable secret admin123

Switch(config)# end

Switch# write memory

4. On PC0 → Desktop → Command Prompt, Telnet into the switch:

telnet 192.168.1.100

5. Enter the VTY password when prompted: remote123

6. You are now remotely logged into the switch. Enter enable mode:

Switch> enable

Password: admin123

7. Run remote commands on the switch:

Switch# show running-config

Switch# show mac address-table

Switch# show interfaces

Part B — PC Remote Desktop Simulation

In Packet Tracer, ensure both PCs have IPs and are connected to the switch.

On PC1 — go to Desktop → Remote Desktop → Enable Remote Desktop (set a password).

On PC0 — go to Desktop → Remote Desktop.

Enter PC1's IP address (192.168.1.11) and the password you set.

Click Connect — PC0 now shows PC1's Desktop interface remotely.

You can interact with PC1's desktop from PC0, simulating real remote desktop access.

📝 Note: Telnet is insecure (sends passwords in plain text). In real networks, SSH is preferred. In Packet Tracer, you can configure SSH on routers/switches using 'crypto key generate rsa' and 'transport input ssh' commands.

End of Practical Manual | Computer Networks | NEP UGCF 2022 | Academic Year 2024-25